YARN MASTER – ERP DEVELOPER HANDOVER DOCUMENT

Functional Specification • UI • Database • Yarn Count • Ply • Spinning • Dyeing • Effects

# 1. Purpose

This document defines the Yarn Master required for the ERP. Yarn must be maintained as a structured master instead of storing descriptions such as 30s/30s/10s, 34s/34s/10s or 40s/40s/20s as uncontrolled text. The system must separately maintain fibre, blend, spinning system, count, ply, yarn construction, twist, dyeing, colour and special yarn effects.

# 2. Core Design Principle

| Legacy Yarn Description | New ERP Structure |
| --- | --- |
| 30s / 30s / 10s | Count sequence: 30s + 30s + 10s; construction/ply rules stored separately |
| 34s / 34s / 10s | Count sequence: 34s + 34s + 10s |
| 40s / 40s / 20s | Count sequence: 40s + 40s + 20s |
| 60s / 60s / 20s | Count sequence: 60s + 60s + 20s |
| 2/50s | Ply = 2; Count = 50s |
| 50/1 | Ply = 1; Count = 50s |
| 100% Cotton 30s | Fibre = Cotton 100%; Count = 30 Ne |
| Cotton/Polyester 30s | Composition child rows + Count + Spinning System |

# 3. Yarn Master Screen

| Section | Fields | Input Type |
| --- | --- | --- |
| Basic | Yarn Code, Yarn Name, Yarn Category, Status | Text / Dropdown |
| Fibre Composition | Fibre + Percentage, multiple rows | Child Grid |
| Spinning | Spinning System, Spinning Method | Dropdown |
| Count | Count System, Count Value, Count Unit | Dropdown + Numeric |
| Ply | Ply / Fold Count | Integer |
| Yarn Construction | Single / Plied / Cabled / Core etc. | Dropdown |
| Twist | Twist Direction, TPI/TPM | Dropdown + Numeric |
| Yarn Effect | Slub, Neppy, Melange, Grindle etc. | Multi-select |
| Dyeing | Yarn Dyed / Piece Dyed / Natural / Dope Dyed etc. | Dropdown |
| Colour | Colour Code, Colour Name, Pantone where applicable | Lookup |
| Technical | Strength, Elongation, Hairiness, CV%, Moisture etc. | Numeric |
| Traceability | Supplier, Lot, Batch, Origin | Lookup / Text |
| Description | Auto Generated Yarn Description | Read-only |
| Documents | Spec Sheet / Test Certificate | File Upload |

# 4. Yarn Category Master

| Code | Yarn Category |
| --- | --- |
| YCAT-001 | Spun Yarn |
| YCAT-002 | Filament Yarn |
| YCAT-003 | Blended Yarn |
| YCAT-004 | Core Spun Yarn |
| YCAT-005 | Covered Yarn |
| YCAT-006 | Fancy / Effect Yarn |
| YCAT-007 | Elastic Yarn |
| YCAT-008 | Recycled Yarn |
| YCAT-009 | Specialty / Technical Yarn |

# 5. Fibre Composition – Child Table

Fibre composition must be stored as child rows and validated to 100%.

| Seq | Fibre | Percentage | Example |
| --- | --- | --- | --- |
| 1 | Cotton | 100% | 100% Cotton |
| 1 | Cotton | 70% | 70% Cotton |
| 2 | Polyester | 30% | 30% Polyester |
| 3 | Elastane | 5% | 5% Elastane |

Allow 1–N fibre components.

Percentage must be numeric and between 0 and 100.

Total composition must equal exactly 100% before Save.

Fibre must come from Fibre Master.

Keep certification attributes separately, e.g. Organic, BCI, Recycled, GOTS/GRS where applicable and verified.

# 6. Spinning System Master

| Code | Spinning System | Typical Use |
| --- | --- | --- |
| SPN-001 | Ring Spun | General spun yarn |
| SPN-002 | Compact Spun | Low hairiness / premium spun yarn |
| SPN-003 | Open-End (OE) | Coarser / economical spun yarn |
| SPN-004 | Air-Jet Spun | High productivity spun yarn |
| SPN-005 | Rotor Spun | OE/rotor yarn |
| SPN-006 | Vortex Spun | Vortex yarn |
| SPN-007 | Recycled Spun | Recycled fibre yarn |
| SPN-008 | Filament | Filament yarn |

# 7. Count System Master

| Code | Count System | Meaning / ERP Use |
| --- | --- | --- |
| CNT-001 | Ne | English Cotton Count – common for cotton spun yarn |
| CNT-002 | Nm | Metric count |
| CNT-003 | Tex | Mass per 1000 m |
| CNT-004 | dtex | Mass per 10,000 m |
| CNT-005 | Denier | Mass per 9,000 m; commonly used for filament |

The Count System must be explicitly stored. Do not assume that '30s' always means the same count system when data can originate from different suppliers.

# 8. Count & Ply Model

| Field | Example | Meaning |
| --- | --- | --- |
| Count System | Ne | English cotton count |
| Count Value | 30 | Base count |
| Count Display | 30s | User-facing notation |
| Ply | 1 | Single yarn |
| Ply | 2 | Two-ply yarn |
| Fold / Ply Count | 2 | Two component yarns |
| Count Sequence | 30s / 30s / 10s | Multiple component count information |

Store numeric count separately from the display suffix such as 's'.

Store the count system separately from the count value.

Ply must be numeric.

For multi-component yarn descriptions, use a child Yarn Component table rather than one free-text field.

Do not mathematically reinterpret a supplier's 30s/30s/10s notation unless the yarn specification confirms the construction.

# 9. Yarn Component Child Table – Recommended

| Seq | Component Count | Count System | Component Fibre | Component Role |
| --- | --- | --- | --- | --- |
| 1 | 30 | Ne | Cotton | Main |
| 2 | 30 | Ne | Cotton | Main |
| 3 | 10 | Ne | Cotton | Binder / Special component |

This model allows the ERP to represent complex yarn specifications such as 30s/30s/10s without losing the original component-level information.

# 10. Yarn Construction Master

| Code | Construction |
| --- | --- |
| YCN-001 | Single |
| YCN-002 | Plied |
| YCN-003 | Cabled |
| YCN-004 | Core Spun |
| YCN-005 | Covered |
| YCN-006 | Twisted |
| YCN-007 | Fancy / Effect |
| YCN-008 | Textured Filament |

# 11. Yarn Effect / Special Yarn Master

| Code | Effect |
| --- | --- |
| YEF-001 | Slub |
| YEF-002 | Neppy |
| YEF-003 | Melange |
| YEF-004 | Grindle |
| YEF-005 | Marl |
| YEF-006 | Space Dyed |
| YEF-007 | Mélange / Heather |
| YEF-008 | Fancy Stripe |
| YEF-009 | Flame |
| YEF-010 | Core Effect |

# 12. Dyeing / Colour Master

| Field | Suggested Values |
| --- | --- |
| Yarn Dye Status | Undyed / Natural / Yarn Dyed / Piece Dyed / Dope Dyed |
| Dyeing Method | Reactive / Disperse / Vat / Pigment / Other approved method |
| Colour Code | ERP Colour Code |
| Colour Name | Approved Colour Master |
| Pantone | Optional reference where applicable |
| Shade / Lot | Supplier shade and lot information |

# 13. Twist & Technical Parameters

| Field | Example | Rule |
| --- | --- | --- |
| Twist Direction | S / Z | Dropdown |
| TPI | 18.5 | >= 0 |
| TPM | 730 | Optional alternate unit |
| Strength | Numeric | Unit required |
| Elongation % | Numeric | 0–100 |
| Hairiness | Numeric | Supplier/test specification |
| CV % | Numeric | Supplier/test specification |
| Moisture % | Numeric | 0–100 |

# 14. Yarn Certification / Sustainability Attributes

| Attribute | Examples |
| --- | --- |
| Organic | Organic Cotton |
| BCI | BCI Cotton |
| Recycled | Recycled Cotton / Recycled Polyester |
| Certified Recycled | GRS where verified |
| Other Certification | GOTS / OCS / OEKO-TEX etc. where verified |

Certification must be represented as an attribute with certificate/reference details where required; do not encode it only in the yarn name.

# 15. Auto Yarn Description Logic

The ERP should generate a standardized description from structured attributes.

| Input | Example |
| --- | --- |
| Composition | 100% Cotton |
| Certification | BCI |
| Spinning | Compact Spun |
| Count | 30 Ne |
| Ply | 1 |
| Effect | Slub |
| Dyeing | Yarn Dyed |
| Generated Description | 100% COTTON BCI COMPACT SPUN 30 NE 1PLY SLUB YARN DYED |

Generated Description is read-only by default.

Use one approved ordering rule for all yarns.

Do not create duplicate spaces or blank-value tokens.

Retain Original / Supplier Description separately when required.

# 16. Sample Yarn Records

| Supplier / Legacy Description | Structured ERP Values |
| --- | --- |
| 30s / 30s / 10s | Component Counts: 30 Ne / 30 Ne / 10 Ne; component-level fibre and role to be captured from specification |
| 34s / 34s / 10s | Component Counts: 34 Ne / 34 Ne / 10 Ne; component-level fibre and role to be captured |
| 40s / 40s / 20s | Component Counts: 40 Ne / 40 Ne / 20 Ne; component-level fibre and role to be captured |
| 60s / 60s / 20s | Component Counts: 60 Ne / 60 Ne / 20 Ne; component-level fibre and role to be captured |
| 2/50s Cotton | 100% Cotton ¦ 50 Ne ¦ 2 Ply |
| 1/40s Cotton | 100% Cotton ¦ 40 Ne ¦ 1 Ply |
| 50s Cotton Slub | 100% Cotton ¦ 50 Ne ¦ Slub ¦ Spinning system to be selected |

# 17. Database Model

| Table | Key Fields |
| --- | --- |
| yarn_master | yarn_id, yarn_code, yarn_name, category_id, spinning_id, count_system_id, count_value, ply, construction_id, twist_direction, tpi, tpm, dyeing_id, colour_id, status, generated_description, supplier_description |
| yarn_composition | yarn_id, seq_no, fibre_id, percentage |
| yarn_component | yarn_id, seq_no, component_count, count_system_id, fibre_id, component_role, component_ply |
| yarn_effect | yarn_id, effect_id |
| yarn_certification | yarn_id, certification_id, certificate_no, validity_from, validity_to |
| yarn_lot | yarn_id, supplier_id, lot_no, batch_no, shade_no, received_date |
| fibre_master | fibre_id, fibre_code, fibre_name, active |
| spinning_master | spinning_id, code, name, active |
| count_system_master | count_system_id, code, name, active |
| yarn_construction_master | construction_id, code, name, active |
| yarn_effect_master | effect_id, code, name, active |

# 18. Mandatory Validations

Yarn Code must be unique.

Yarn Category is mandatory.

At least one fibre composition row is required for normal spun/blended yarns.

Composition total must equal 100%.

Count System is mandatory when a count is provided.

Count Value must be greater than zero.

Ply must be a positive integer where applicable.

Count sequence must be stored component-wise for multi-component descriptions.

Spinning System must be selected for spun yarns.

Filament yarns should use Denier/dtex/Tex as applicable rather than assuming Ne.

TPI/TPM cannot be negative.

Inactive master values cannot be selected for new transactions.

Duplicate yarn combinations should trigger a warning.

Unknown supplier codes/descriptions must be flagged for review, not guessed.

# 19. Yarn Master List Screen

| Column | Purpose |
| --- | --- |
| Yarn Code | Unique searchable code |
| Yarn Description | Generated standard description |
| Composition | Short composition |
| Spinning | Ring / Compact / OE / etc. |
| Count | 30 Ne / 50 Ne etc. |
| Ply | 1 / 2 / 3 etc. |
| Construction | Single / Plied / Core Spun etc. |
| Effect | Slub / Melange / etc. |
| Dyeing | Yarn Dyed / Undyed etc. |
| Status | Active / Inactive |
| Actions | View / Edit / Copy / Inactivate |

# 20. Search & Filter

Yarn Code / Yarn Description

Fibre / Composition

Spinning System

Count System

Count Range

Ply

Yarn Construction

Yarn Effect

Dyeing Method

Colour

Certification

Supplier / Lot

Active / Inactive

# 21. User Workflow

Add New → Select Yarn Category → Add Fibre Composition → Select Spinning System → Select Count System → Enter Count → Enter Ply / Components → Select Yarn Construction → Enter Twist → Select Effect → Select Dyeing / Colour → Add Certification if applicable → Enter Technical Parameters → Review Auto Description → Save.

# 22. Fabric Master Integration

Yarn Master must be directly reusable from Fabric Master. A fabric should select one or more approved yarn records instead of typing yarn specifications manually.

| Fabric Requirement | Yarn Master Link |
| --- | --- |
| Warp Yarn | yarn_id |
| Weft Yarn | yarn_id |
| Face Yarn | yarn_id |
| Back / Loop Yarn | yarn_id |
| Plated Yarn | yarn_id |
| Rib Yarn | yarn_id |
| Collar / Cuff Yarn | yarn_id |

Fabric Yarn Mapping should support role/position so the same Yarn Master can be reused.

Fabric costing and consumption should pull the approved yarn specification and rate from the linked yarn/item records.

# 23. Migration of Existing Yarn Data

Legacy yarn descriptions should first be imported into a staging table. The parser should identify count, ply, fibre, spinning, effect and dyeing only where the mapping is reliable.

| Migration Status | Meaning |
| --- | --- |
| Auto Mapped | All required attributes confidently identified |
| Partially Mapped | Some attributes identified; remaining values need review |
| Pending Review | Ambiguous notation such as 30s/30s/10s without specification |
| Duplicate | Equivalent structured yarn already exists |
| Rejected / Other | Not a yarn master record |

# 24. Important Rule for 30s/30s/10s Type Notation

Do not simply save '30s/30s/10s' as Yarn Name. Treat it as a structured multi-component specification. The exact technical interpretation (component role, ply/fold relationship and resulting yarn count) must come from the supplier spinning specification. The ERP should preserve the original notation and component values without silently calculating or changing it.

# 25. Developer Acceptance Criteria

Developer can create a yarn with 1–N fibre composition rows.

System prevents Save when composition total is not 100%.

Count System and Count Value are stored separately.

Ply is stored as a numeric field.

Multi-component yarns are supported through Yarn Component child rows.

30s/30s/10s, 34s/34s/10s, 40s/40s/20s and 60s/60s/20s can be stored without loss of information.

Spinning System is master-driven.

Yarn Effect, Dyeing and Certification are maintained independently.

Auto Description updates from structured values.

Fabric Master can link to Yarn Master by yarn_id.

Inactive yarns cannot be selected in new transactions.

Duplicate detection works before Save.

Legacy/Supplier Description is retained for audit and migration.

# 26. Recommended Final Hierarchy

Yarn Category → Fibre Composition → Spinning System → Count System → Count → Ply / Components → Yarn Construction → Twist → Effect → Dyeing → Colour → Certification → Technical Parameters → Auto Description

END OF YARN MASTER DEVELOPER HANDOVER